EOCap4Africa – 7 Vector Analysis

Sample Solution

Project Setup

• Start QGIS and create a new project file.

• Get a background map using Web -> QuickMapServices -> OSM ->  OSM Standard.

• Load in the clip shapefile for later.

Data Preparation

• First, we want to extract wetlands via OSM. Open QuickOSM (Vector >  QuickOSM) and enter the following:

o Key: natural

o Value: wetland

o Location: Botswana

• Run the query. This may take some time, so be patient.

• Next, clip the wetlands to the predefined study area using the clip.shp: o Open Vector > Geoprocessing Tools > Clip

o Input Layer: Extracted wetlands

o Clip Layer: clip.shp

o Click Run

- Now, extract the road network in the area using OSM:

o If we know our study area, we can directly extract OSM data only  within our area of interest (AOI).

o In QuickOSM, use the clip shapefile in the OSM location tab. o Run the query.

Tip! To keep QGIS efficient, remove shapefiles you will no longer use. OSM  outputs multiple types of data, but for this analysis, we only need:

• The extent of our study area

• The wetlands as polygons

• The roads as a line layer

I also adjusted the visualization of the shapes to improve readability and  renamed the files for easier data handling. My cleaned-up file looks like  this:

Vector Analysis in QGIS

• To buffer data properly, you need to reproject the data to a UTM  projection to perform accurate calculations.

• Save the wetlands polygon as a new shapefile with the correct  coordinate system:

o Right-click the wetlands layer > Export > Save Features As… o Choose a UTM coordinate system appropriate for Botswana o Click Run

• Apply the buffer to the dataset:

o Open Vector > Geoprocessing Tools > Buffer

o Input Layer: Reprojected wetlands

o Distance: 5000 meters (5 km)

o Click Run

o Buffer creation might take a moment, depending on your PC’s  performance.

• Ensure that the buffer correctly expands around the wetlands before  proceeding:

• Last, extract roads that are too close to the buffer using the Intersection  tool:

o Open Vector > Geoprocessing Tools > Intersect

o Input Layer: Road network

o Overlay Layer: Buffered

o Click Run

Visualizing the Results

• Open Project > New Print Layout, name it, and click OK.

• Add the map by selecting Add Map and drawing a rectangle in the  layout.

• Add the following elements using Add Item:

o Title: Use a clear, descriptive name.

o North Arrow: Found under Item Properties.

o Scale Bar: Adjust placement and units.

o Legend: Ensure symbology is properly explained.

o Source & Date: Add a reference to the data sources.