EOCap4Africa – 8b Raster Preprocessing

Sample solution - Mosaicking

Project setup

- Start your QGIS application and create a new project file.

- Get a background map using Web -> QuickMapServices -> OSM -> OSM-Standard.

Load Sentinel-2 bands into QGIS

- Navigate to the Sentinel-2 folder (.SAFE), where you should select the following bands from this location: [your folder structure]\Mosaicking_1\GRANULE\L2A_T38KQD_A002278_20250211T071233\IMG_DATA\R20m:

    - T38KQD_20250211T071231_B02_20m -> B02 (Blue)

    - T38KQD_20250211T071231_B03_20m -> B03 (Green)

    - T38KQD_20250211T071231_B04_20m -> B04 (Red)

Create a virtual raster

- Next, we can merge the 3 bands in one cohesive raster image by using the tool “Build Virtual Raster” in the miscellaneous tab of the raster tools.

- Input the three bands and select “Highest” for resolution.

- Don’t forget to tick the box that ensures that all input layers will be treated as individual bands.

- Repeat this exact process from “Load in Sentinel-2 Bands in QGIS” for the second dataset Mosaicking_2.

Creating the mosaic

- Finally, under raster, go to miscellaneous again and pick the merge tool

- Simply input the two layers you previously created.

- Set the output no_data values to 0. Setting no_data values to zero makes the dark borders disappear.

- Lastly, make sure to save the new file as a .tif with a name of your choice before you run the tool.

Sample solution – Cloud cover

Project setup

- Start your QGIS application and create a new project file.

- Get a background map using Web -> QuickMapServices -> OSM -> OSM-Standard.

Load Sentinel-2 bands into QGIS

- First, there are two Raster datasets that we need to add to our QGIS Project: The raster layer that contains the cloud classification and the raster layer in which we want the cloud to be covered.

- Navigate to the Sentinel-2 folder (.SAFE)., where you should select the following bands from this location:

- [your folder structure] 8b Raster Preprocessing\CloudCover\GRANULE\L2A_T38KQD_A002278_20250211T071233\IMG_DATA\R20m and load in T38KQD_20250211T071231_B03_20m.jp2 -> Green Band

- [your folder structure] \8b Raster Preprocessing\CloudCover\GRANULE\L2A_T38KQD_A002278_20250211T071233\QI_DATA and load in MSK_CLDPRB_20m.jp2 -> Cloud Mask Band

- For the first input, you could technically also choose any other band or a combination of multiple bands built as a virtual raster, as seen before in “Create Virtual Raster”

The two bands loaded in QGIS should look like this in your layer view:

Load Sentinel-2 bands in the band set tool for SCP

- Next, we need to go to the Semi-Automatic Classification Plugin (SCP) and select the Band Set tool.

- The SCP Plug-In can only use bands that have first been added via the Band Set Tool.

- In it, we can select to add bands from our folder:

- Add one of the Bands to a Band Set by clicking on the grey plus symbol in the upper right corner and selecting one of the bands.

- The selected band will show up in the main window. Now, you need to select “Create virtual raster of band set” at the bottom.

- Afterwards, you can run the tool and select any folder of your liking.

- To add a second band set for the remaining raster layer, click on the “Add a new band set” button in the bottom left (symbol with stacked layers and a plus sign).

- Repeat steps 1-3 for the second raster layer.

- In your band set table, two band sets with one raster layer each should now appear:

Creating the cloud mask with the masking bands tool

- The Masking Bands tool can be found under SCP -> Preprocessing -> Masking Bands.

- First, you will need to select the input band set. This is the band that the mask will be applied to; in our case, that's Band 3.

- For me, this raster layer was saved in band set 1, but it depends on the order in which you completed the last step, so be careful here.

- To select the classification, click on the refresh button on the right. Your MSK_CLD raster should automatically be selected. This is the layer that contains the cloud data from which the mask will be created.

- For mask class values, choose 85-100 (based on inspection of your cloud mask layer using the identity tool).

- Select Create virtual raster output before running the tool and selecting a folder of your choice.

Visually inspecting the result

With an OSM map in the background, you should be able to see cloud-shaped holes in your raster like this: